## Flow Control

| Stop and Wait Protocol | Stop and Wait ARQ |
| --- | --- |
| It assumes that the communication channel is perfect and noise free. | It assumes that the communication channel is imperfect and noisy. |
| Data packet sent by the sender can never get corrupt. | Data packet sent by the sender may get corrupt. |
| There is no concept of negative acknowledgements. | A negative acknowledgement is sent by the receiver if the data packet is found to be corrupt. |
| There is no concept of time out timer. | Sender starts the time out timer after sending the data packet. |
| There is no concept of sequence numbers. | Data packets and acknowledgements are numbered using sequence numbers. |

| Aspect/Property | Stop and Wait ARQ | Go Back N | Selective Repeat | Remarks |
| --- | --- | --- | --- | --- |
| Efficiency | 1 / (1+2a) | N / (1+2a) | N / (1+2a) | Go back N and Selective Repeat gives better efficiency than Stop and Wait ARQ. |
| Window Size | Sender Window Size = 1 Receiver Window Size = 1 | Sender Window Size = N Receiver Window Size = 1 | Sender Window Size = N Receiver Window Size = N | Buffer requirement in Selective Repeat is very large. If the system does not have lots of memory, then it is better to choose Go back N. |
| Minimum number of sequence numbers required | 2 | N+1 | 2 x N | Selective Repeat requires large number of bits in sequence number field. |
| Retransmissions required if a packet is lost | Only the lost packet is retransmitted | The entire window is retransmitted | Only the lost packet is retransmitted | Selective Repeat is far better than Go back N in terms of retransmissions required. |
| Bandwidth Requirement | Bandwidth requirement is Low | Bandwidth requirement is high because even if a single packet is lost, entire window has to be retransmitted. Thus, if error rate is high, it wastes a lot of bandwidth. | Bandwidth requirement is moderate | Selective Repeat is better than Go back N in terms of bandwidth requirement. |
| CPU usage | Low | Moderate | High due to searching and sorting required at sender and receiver side | Go back N is better than Selective Repeat in terms of CPU usage. |
| Level of difficulty in Implementation | Low | Moderate | Complex as it requires extra logic and sorting and searching | Go back N is better than Selective Repeat in terms of implementation difficulty. |
| Acknowledgements | Uses independent acknowledgement for each packet | Uses cumulative acknowledgements (but may use independent acknowledgements as well) | Uses independent acknowledgement for each packet | Sending cumulative acknowledgements reduces the traffic in the network but if it is lost, then the ACKs for all the corresponding packets are lost. |
| Type of Transmission | Half duplex | Full duplex | Full duplex | Go back N and Selective Repeat are better in terms of channel usage. |

# 2. Error Control

CRC -  Cycle Redundancy Check

Rules for CRC

Rule-01:

- It should not be divisible by x.

- This condition guarantees that all the burst errors of length equal to the length of polynomial are detected.

Rule-02:

- It should be divisible by x+1.

- This condition guarantees that all the burst errors affecting an odd number of bits are detected.

Important Notes-

If the CRC generator is chosen according to the above rules, then-

- CRC can detect all single-bit errors

- CRC can detect all double-bit errors provided the divisor contains at least three logic 1’s.

- CRC can detect any odd number of errors provided the divisor is a factor of x+1.

- CRC can detect all burst error of length less than the degree of the polynomial.

- CRC can detect most of the larger burst errors with a high probability.

Refer Formula Notes

# Access Control

CSMA/CD

# Switching

# Routing

## IPv4

216.27.61.137 = 11011000.00011011.00111101.10001001

## Classes

Class A

2^8 is 256 Networks

2^24 is 1, 67, 77, 216 hosts/subnets

24+8=32

Used for large organizations, e.g., CISCO.

x.x.x.x/8 – default mask of 8-bits in Network.

1st bit is ﬁxed to 0 and 2 special addresses are ﬁxed (first and last),

- therefore 2^7 (8-1 as first bit is fixed) Networks

- 2^24– 2 hosts (-2 for start and end address)

## Subnetting

### First Address/ Network Address

The ﬁrst address in the block can be found by setting the (32-n) right most bits in the binary notation of the address to Zeros.

Alternatively, the ﬁrst address can be found by ANDing the given address with the mask.

Address: 11010001.00010001.00100110.00101000

Mask :  11111111.11111111.11111111.11110000

### Last address/Broadcast Address/Limited Broadcast Address

The last address in the block can be found by setting the (32 – n) right most bits in the binary notation of the address to 1’s.

Another way to ﬁnd the last address is by OR-ing the given address with the complement of the mask.

### Total Number of Addresses

The number of addresses in the block is the difference between the last and ﬁrst address.

It can be found using formula 2^(32-n)

The number of addresses can also be found by complementing the mask, interpreting it as a decimal number, and adding 1 to it.

Mask Complement:

00000000.00000000.00000000.00001111

Number of addresses: 15 + 1 = 16

(complement of a number is found by changing each 1 to 0 and each 0 to 1)

# IPv4 Header

# TCP

# Application Layer

|  | DNS | HTTP | SMTP | POP | FTP |
| --- | --- | --- | --- | --- | --- |
| Stateful / Stateless | Stateless | Stateless | Stateless | Stateful | Stateful |
| Transport Protocol Used | UDP | TCP | TCP | TCP | TCP |
| Connectionless / Connection Oriented | Connectionless | Connectionless | Connection Oriented | Connection Oriented | Connection Oriented |
| Persistent / Non-persistent | Non-persistent | HTTP 1.0 is non-persistent. HTTP 1.1 is persistent. | Persistent | Persistent | Control connection is persistent. Data connection is non-persistent. |
| Port Number Used | 53 | 80 | 25 | 110 | 20 for data connection. 21 for control connection. |
| In band / Out-of-band | In band | In band | In band | In band | Out-of-band |

# Network Security